Street Stock and Hobby Stock Safety Check List

___	No fluid reservoirs, battery, radiators in driver compartment

___	Good metal firewalls between driver and engine plus driver and trunk with no holes

___	Good racing driver seat mounted firmly to frame (not floor)

___	Five point seat belts mounted properly, meets SFI 16.1 and less than 3 years old

___	Six point roll cage, gusseted and welded to the frame. Min. 1 ½” o.d. tubing.

___	Minimum of 4 bars of 1 1/2” diameter in driver door.  Three bars in pass. door. Minimum thickness of bars is .095”.

___	Two 1 ½” bars must go rearward thru firewall to rear frame in Street Stocks

___	Roll cage padding around areas where driver may come into contact

___	Protective windscreen with at least two metal rods near center of windshield

___	No open fuel lines in drivers compartment (Can be encased and sealed at both ends)

___	Front and rear tow hooks capable of supporting cars weight

___	Racing fuel cell in a steel box supported by two 1/8” X 2” steel straps

___	If car is a unibody, the frame must be tied together in a sound way

___	Standard shift cars must have a blow proof bellhousing

___	Drive shaft painted white with a safety loop in front.  Hoop must be min. 6” in dia. By 1” wide and ¼” thick

___	Mufflers installed

___	Good driver fire suit, meets SFI 3.2  and less than 5 years old.

___	Full faced helmet Snell 90, 95, K98 , 2000 or newer

___	Oversized lug nuts and studs (1/2” dia. Minimum)

___	Any ballast must have a minimum of 2 mounting bolts a minimum of ½” diameter.  All must be painted white with car number on them